ALTERNATING CODING AND EFFICIENT DECODERS DESIGNED ON THE BASIS OF IT FOR IMAGE CODING

Mid-Sweden University, Dept. of Information Technology and Media, Electronics Design Division

Shang Xue and Bengt Oelmann

Introduction

The image data such as the motion-vector and the transformed coefficients can usually be modelled by the Generalized Gaussian (GG) distribution and are then coded using Exp-Golomb (EG) coded or Golomb-Rice (GR) code. GR and EG codes are both nearly optimum for GG sources. For instance, Universal Variable Length Code (UVLC), which is used universally in the coding of all image data types in H.26L, is actually a reversible EG code. This work studies the coding characteristics of image data (GG sources). Based on this study, another nearly optimal code -- Hybrid Golomb code (HG), as well as an efficient coding method -- Alternating Coding (ALT) are proposed for GG shaped data sources. Efficient decoders are then designed on the basis of ALT coding. ALT coding is applied to the GR code and the EG code. Results show that ALT coded UVLC packet conceives same coding efficiency, whereas the error resiliency and robustness are improved. Moreover, UVLC and GR decoders designed on the basis of the ALT coding are greatly enhanced in terms of speed, area and power consumption.

Normal UVLC

ALT coding

Alt coding and image decoders based on it

The basic idea of ALT coding is to separates the code into two parts, one part contains the code length information, while another can be determinded by the first part. The first part is then coded using two code tables alternatingly with the codeword boundaries obviously marked. The two parts of codes are then transimitted separately. Take UVLC as an example. Table 1 shows an example of UVLC.

##### Table 1. An example of UVLC

| Class | UVLC | Length | Value to be expressed |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |
| 2 | 0x00 | 3 | ’x0’+ 2[2:3] |
| 3 | 0x11x00 | 5 | ’x1x0’+ 4[4:7] |
| 4 | 0x21x11x00 | 7 | ’x2x1x0’+ 8[8:15] |
| … | … … | … | … … |

Here, odd-indexed bits (OIB) of each UVLC can be represented in a unary way which contains code length information. The OIBs are then coded using either {1, 11, 111, 1111, …} or {0, 00, 000, 0000, …}, in an alternating way. Its even-indexed bits (EIB) is an arbitrary binary code whose length is determined by its corresponding OIB. The whole packet is then transmitted as Figure 1 illustrates.

… … …

… … …

Sync.

Marker

Sync.

Marker

OIB sequence

EIB sequence

Sync.

Marker

UVLC1

UVLC2

UVLC3

… …

UVLCn

Sync.

Marker

OIB1

OIB2

OIB3

OIBn

EIB1

EIB2

EIB3

EIBn

Figure 1. ALT coding for UVLC

The advantages of ALT:

- Immediate code boundary detection (hardware/software)

- Easy error detection and location (software)

The ALT decoders, obviously, should have two parts, one decoder decodes each code length (this can be done in parallel), and another decodes the rest of the code.

Results & Future work

Simulations were conducted to demonstrate the error resilience of ALT coding and the efficiency of the ALT decoders.

- Error resiliency demonstrated by Correct Ratio (CR)

- Visual quality of ALT coded images

Table 2. Comparison of PSNR of the UVLC decoders

| Image | PSNR of UVLC (dB) | PSNR of ALT (dB) |
| --- | --- | --- |
| Lena | 21.92 | 27.50 |
| Cameraman | 24.23 | 3146 |
| Monkey | 17.81 | 22.38 |
| House | 27.67 | 30.07 |

- VLSI Implementation of ALT decoders

Table 3. Comparison of performance of the UVLC decoders

|  | ALT | PLS | Ratio (ALT/PLS) |
| --- | --- | --- | --- |
| Delay (ns) | 8.96 | 12.0 | 75% |
| Area (gates) | 1855 | 3146 | 59% |
| Power (mW) | 6.74 | 15.0 | 45% |

Table 4. Comparison of performance of the GR* decoders

|  | ALT | PLS | Ratio (ALT/PLS) |
| --- | --- | --- | --- |
| Delay (ns) | 8.91 | 10.15 | 88% |
| Area (gates) | 736 | 1441 | 51% |
| Power (mW) | 3.9 | 13.8 | 28% |

GR with one bit suffix

Future work will be focused on further exploring new codes for GG sources and applying ALT coding to them.